Title: DesIgn, AnalysIs, and ManufacturIng of a Permanent Magnet Synchronous Generator (PMSG) for Small Scale WInd TurbInes

Ref Code	:	PMSG (Acronym)

Type	: 	New

Status	:	Individual

Vacancy	: 	N/A

Duration	:	1 Semester

Project Description:

Small-scale wind energy harvesting is a key component of decentralized renewable energy systems. This project aims to design and manufacture a low-cost Permanent Magnet Synchronous Generator (PMSG) by retrofitting a standard single-phase AC induction motor. The main objective is to convert the asynchronous motor structure into a synchronous generator. This will be accomplished by machining the original rotor core to create slots for embedding Neodymium (NdFeB) magnets. This embedded configuration provides mechanical retention against centrifugal forces and ensures the rotor initiates rotation even at low wind speeds. The project emphasizes engineering analysis and simulation to validate the electromagnetic and mechanical design before physical prototyping. The final deliverable will be a functional generator capable of charging a standard 12 V battery, with performance data compared to the analytical models.

Work Description:

- Conduct a literature survey on PMSG design and induction motor retrofitting techniques.

- Disassemble the motor and measure stator/rotor geometry for CAD modeling.

- Perform electromagnetic simulations (ANSYS) to optimize magnet placement and reduce cogging   torque.

- Conduct structural stress analysis on the modified rotor to ensure a safety factor > 2 under high RPMs.

- Machine the existing rotor on a lathe/mill to create recesses and embed the magnets.

- Rewind the stator coils to match the target voltage/current requirements.

- Construct a test bench to drive the generator and perform the following experiments:

- Compare the experimental efficiency results with the FEA simulation data.

Required Qualifications and Skills:

- Knowledge of Machine Design and Electromechanical Energy Conversion.

- Proficiency in CAD software (SolidWorks, Onshape, Fusion 360, CATIA, etc.).

- Familiarity with Engineering Simulation tools (ANSYS Workbench, Maxwell, or FEMM).

- Hands-on experience with machining (lathe/milling) and electrical measurements.